University of Information Technology – VNU-HCM

Faculty of Computer Engineering

Computer System

LAB 01

Getting to Know MARS: Registers, Arithmetic Instructions and System Calls

| Full name | Phan Trung Kiên | Student ID | 26560035 |
| --- | --- | --- | --- |
| Class | CSBU102.R11.KHBC | Practice date | 28/10/2026 |
| Self-assessed score |  | GitHub link |  |

1. LAB OVERVIEW

Purpose. In this session you will meet the machine that sits underneath every program you have ever used. Starting from an ordinary arithmetic statement such as a = b + c, you will see how a processor keeps numbers in registers, how one assembly instruction works on three registers, how that instruction is stored as a single 32-bit number, and how a running program talks to the keyboard and the screen through system calls. Every new idea is explained first and then practised.

Situation: the arithmetic inside a smart electricity meter A household electricity meter keeps a running total of the energy used, measured in kilowatt-hours (kWh). The electricity company notes the meter reading at the start of each month. The energy used in a month is the difference between two consecutive readings, and the energy used over several months is the sum of the monthly differences. The small processor inside a smart meter carries out exactly these subtractions and additions on numbers held in registers, and then reports the results. All readings used in this LAB are fictitious.

After this session, you can:

- open, assemble, run, step through and reset a program in MARS, and find the Registers, Text Segment, Data Segment and Run I/O panes;

- name the main MIPS registers ($zero, $t0–$t9, $s0–$s7, $v0, $a0, pc) and read their contents in decimal and hexadecimal;

- translate simple arithmetic statements such as a = b + c into MIPS instructions that use add, sub and addi, with registers playing the role of variables;

- describe how an assembly instruction is stored as a 32-bit machine-code word and read the register numbers inside it;

- use system calls to print text and integers and to read an integer from the keyboard;

- predict, observe and explain the behaviour of a short assembly program.

Table 1. Structure of the session

| Part | Content | Suggested time | Score |
| --- | --- | --- | --- |
| 0 | Getting started with MARS | 20 min | – |
| 1 | Experiment 1 – Meet the registers | 30 min | 20 |
| 2 | Experiment 2 – From a = b + c to $t0 = $t1 + $t2 | 45 min | 30 |
| 3 | Experiment 3 – Input, output and system calls | 50 min | 35 |
| 4 | Wrap-up and submission | 15 min | 15 |
| – | Debugging, checking and review | 20 min | – |
| Total |  | 180 min | 100 |

Working conventions • A yellow cell is where you write an answer or a value. A dashed frame is where you paste a screenshot. • Program listings appear in grey frames in a monospaced font. In a listing, everything after # is a comment. A line that starts with # >>> marks a place where you write code. • Questions are labelled with the type of thinking they ask for: Understand, Apply, Analyze or Evaluate. • Many activities follow the cycle PREDICT → OBSERVE → EXPLAIN. First write down what you expect (PREDICT), then run the program and record what really happens (OBSERVE), and finally explain any difference (EXPLAIN). A wrong prediction is never penalised; the explanation is what earns the marks. • Tick the box ☐ in a checklist when you have completed the item.

PART 0. GETTING STARTED WITH MARS  (20 minutes)

MARS (MIPS Assembler and Runtime Simulator, version 4.5) is a Java program that imitates a MIPS processor. It lets you run assembly programs and watch registers and memory change one instruction at a time, without any real MIPS hardware.

Table 2. Skills checklist

| ☐ | Skill | Hint |
| --- | --- | --- |
| ☐ | Start MARS and create a new file | Double-click Mars4_5.jar (or run java -jar Mars4_5.jar), then File → New. |
| ☐ | Check the settings used in this LAB | In the Settings menu: Permit extended (pseudo) instructions and formats ticked; Initialize Program Counter to global ‘main’ if defined ticked; Delayed branching unticked; Popup dialog for input syscalls unticked. |
| ☐ | Assemble a program | Run → Assemble (or the toolbar icon). MARS switches from the Edit tab to the Execute tab. |
| ☐ | Run the whole program | Run → Go. Text printed by the program appears in the Run I/O pane at the bottom. |
| ☐ | Execute one instruction at a time | Run → Step runs the next instruction; Run → Backstep undoes one; Run → Reset returns to the start. Shortcut keys are shown next to each command. |
| ☐ | Identify the panes of the Execute tab | Registers (right), Text Segment (upper left), Data Segment (below it), Messages and Run I/O (bottom). |
| ☐ | Switch values between decimal and hexadecimal | Settings → Values displayed in hexadecimal. |
| ☐ | Save your work | File → Save as… and use the file-name rule given below. |

Note MARS has two views of a program: the Edit tab, where you type, and the Execute tab, where you run it. After every change in the editor you must assemble again. Assembling also resets all registers and memory to their initial state.

Step 0 – Quick check

Type the program in Listing 0 into a new file (use the exact text), save it, assemble it and run it. You will learn what each line means in Experiment 3; for now, only confirm that your installation works.

Listing 0

# Listing 0 - quick check .data msg: .asciiz "Hello, MARS!\n" .text .globl main main: li $v0, 4 la $a0, msg syscall li $v0, 10 syscall

Text that appears in the Run I/O pane:

Hello, MARS! -- program is finished running --

File-name rule Every program you submit is saved as Lab01_<StudentID>_<Part>.asm, for example Lab01_22520001_TN3.asm. Use TN2 or TN3 for the part. Start every file with a comment block containing your name, student ID and a one-line purpose.

EXPERIMENT 1. MEET THE REGISTERS  (30 minutes – 20 points)

Situation The processor in a smart meter does not calculate on numbers stored far away in memory. It first brings the numbers into a small set of very fast storage cells called registers and calculates there. Before programming the meter, you need to know the registers.

Background: registers and the program counter A processor executes instructions one after another. MIPS has 32 registers, each 32 bits wide (one word). Every register has a number from 0 to 31 and a name that starts with $. Table 3 lists the registers used in this LAB. The processor also has a special register, the program counter (pc), which holds the address of the next instruction to execute. MARS shows register contents as hexadecimal numbers such as 0x0000002A: each hexadecimal digit stands for 4 bits, so 8 digits make 32 bits.

Table 3. Registers used in this LAB

| Name | Number | Conventional use |
| --- | --- | --- |
| $zero | 0 | The constant 0. It always holds 0. |
| $v0–$v1 | 2–3 | Results; $v0 also carries the service number of a system call (Experiment 3). |
| $a0–$a3 | 4–7 | Arguments; $a0 also carries the value given to a system call (Experiment 3). |
| $t0–$t9 | 8–15, 24–25 | Temporary values. |
| $s0–$s7 | 16–23 | Variables whose values are kept for a longer time. |
| $sp | 29 | Stack pointer (used in a later LAB). |
| $ra | 31 | Return address (used in a later LAB). |

Step 1.1 – Find the registers (8 min)

Assemble Listing 0 but do not run it. Look at the Registers pane and complete Table 4 with the register number and its initial value (hexadecimal).

Table 4. Initial register contents

| Register | Number | Initial value (hex) |
| --- | --- | --- |
| $zero | 0 | 0x00000000 |
| $v0 | 1 | 0x00000000 |
| $a0 | 2 | 0x00000000 |
| $t0 | 3 | 0x00000000 |
| $t9 | 4 | 0x00000000 |
| $s0 | 5 | 0x00000000 |
| $sp | 6 | 0x7fffeffc |
| pc | 32 | 0x00400000 |

Question 1.1 (Understand)  Compare the initial value of pc with the Address column of the first row of the Text Segment. What do you notice, and what does this tell you about the job of the program counter?

The initial value of the pc (0x00400000) matches the Address column in the Text Segment, this indicates that the PC stores the address of the instruction.

Step 1.2 – Predict, observe, explain (15 min)

Open a new file, type Listing 1, save it and assemble it. Before you run anything, fill the PREDICT column of Table 5 with the decimal value that the destination register should hold after each instruction.

Listing 1

# Listing 1 - first steps with registers .text .globl main main: addi $t0, $zero, 12 # $t0 = $zero + 12 addi $t1, $zero, 30 # $t1 = $zero + 30 add $t2, $t0, $t1 # $t2 = $t0 + $t1 addi $zero, $zero, 5 # try to put $zero + 5 into $zero add $t3, $zero, $t2 # $t3 = $zero + $t2 li $v0, 10 # end of program (see Experiment 3) syscall

Now use Run → Step to execute the first five instructions one at a time. After each Step, MARS highlights the register that has just changed. Record the observed value of the destination register and the value of pc after the instruction. Use Backstep at least once to see an instruction being undone.

Table 5. Predict and observe (decimal values for registers, hexadecimal for pc)

| Instruction | Destination register | PREDICT value | OBSERVE value | OBSERVE pc after the instruction |
| --- | --- | --- | --- | --- |
| addi $t0, $zero, 12 | $t0 | 12 | 12 | 0x00400004 |
| addi $t1, $zero, 30 | $t1 | 30 | 30 | 0x00400008 |
| add $t2, $t0, $t1 | $t2 | 42 | 42 | 0x0040000c |
| addi $zero, $zero, 5 | $zero | 5 | 0 | 0x00400010 |
| add $t3, $zero, $t2 | $t3 | 42 | 42 | 0x00400014 |

Question 1.2 (Apply)  Switch the display to hexadecimal. $t2 now shows 0x0000002a. Convert this number to decimal by hand and show your calculation.

2×16+10×1=32+10=42

Question 1.3 (Analyze)  One of your predictions did not match what MARS showed. Which instruction was it? Give your explanation of what happened and what this says about $zero.

Lệnh addi $zero, $zero, 5, dù được gán giá trị 5, $zero vẫn giữ giá trị 0, thanh ghi $zero được thiết kế cứng ở mức phần cứng để luôn luôn mang giá trị 0 và không thể bị thay đổi.

Question 1.4 (Analyze)  By how many does pc increase after each instruction? Every MIPS instruction occupies one 32-bit word in memory. Use this fact to explain the number you observed.

Pc tăng thêm 4bytes, vì mỗi lệnh MIPS là một từ tương ứng với 32bits là 4 bytes, nên pc cần tăng thêm 4bytes sau mỗi lệnh

Screenshot of the Registers pane after the fifth instruction:

Paste screenshot here

EXPERIMENT 2. FROM  a = b + c  TO  $t0 = $t1 + $t2  (45 minutes – 30 points)

Situation The firmware of the meter must compute the energy used between two readings. In a high-level language the programmer writes usage = reading2 - reading1. The processor, however, executes only instructions that name exactly three registers. This experiment shows how a statement is translated step by step, down to the number that the machine really executes.

Background: from a statement to machine code The same operation can be written at three levels (Table 6). The program that translates assembly into machine code is called the assembler; MARS contains one.

Table 6. Three levels of the same operation

| Level | Example | Written by |
| --- | --- | --- |
| High-level statement | a = b + c | the programmer |
| Assembly instruction | add $t0, $t1, $t2 | the programmer (or a compiler) |
| Machine code | one 32-bit number, shown as 8 hexadecimal digits | the assembler |

Variables become registers A processor has no named variables. We decide which register stands for which variable, for example a → $t0, b → $t1, c → $t2. The instruction add has the form add destination, source1, source2 so a = b + c becomes add $t0, $t1, $t2. One instruction performs one operation on three registers; a longer expression needs several instructions. For example, x = p + q - r with x→$t0, p→$t1, q→$t2, r→$t3 needs two instructions: add $t0, $t1, $t2 # x = p + q sub $t0, $t0, $t3 # x = x - r. To put a constant into a register, add it to $zero: addi $t1, $zero, 7 sets $t1 to 7. To copy a register, add $zero to it: add $t2, $t1, $zero copies $t1 into $t2.

Step 2.1 – Your first addition (8 min)

Type Listing 2, save it and assemble it. PREDICT the final value of $t0 before you run it.

Listing 2

# Listing 2 - a = b + c .text .globl main main: addi $t1, $zero, 7 # b = 7 addi $t2, $zero, 5 # c = 5 add $t0, $t1, $t2 # a = b + c li $v0, 10 syscall

PREDICT – final value of $t0 (decimal):

12

OBSERVE – execute the three instructions with Step and write down the three registers after each one.

Table 7. Registers after each instruction of Listing 2

| After executing | $t0 | $t1 | $t2 |
| --- | --- | --- | --- |
| addi $t1, $zero, 7 | 0x00000000 | 0x00000000 | 0x00000000 |
| addi $t2, $zero, 5 | 0x00000000 | 0x00000007 | 0x00000000 |
| add $t0, $t1, $t2 | 0x00000000 | 0x00000007 | 0x00000005 |

Step 2.2 – Look at the machine code (12 min)

Select the Execute tab and look at the Text Segment. Each row has the columns Address, Code, Basic and Source: Source is the text you typed, Basic is the real instruction the processor executes, and Code is the 32-bit machine word in hexadecimal. Find the row of add $t0, $t1, $t2 and note its address and code.

Background: the R-type instruction format Instructions such as add and sub use the R format, in which the 32 bits are divided into six fields: ¦ op ¦ rs ¦ rt ¦ rd ¦ shamt ¦ funct ¦ ¦ 6 bits ¦ 5 bits ¦ 5 bits ¦ 5 bits ¦ 5 bits ¦ 6 bits ¦. op is the operation code; rs and rt are the numbers of the first and second source registers; rd is the number of the destination register; shamt is a shift amount (0 for add); funct selects the exact operation. To read a hexadecimal code, replace every digit by 4 bits: 0=0000, 1=0001, 2=0010, 3=0011, 4=0100, 5=0101, 6=0110, 7=0111, 8=1000, 9=1001, A=1010, B=1011, C=1100, D=1101, E=1110, F=1111.

Address of the row:

0x00400008

Code of the row (hexadecimal):

0x012a4020

Table 8. Fields of the instruction add $t0, $t1, $t2

|  | op | rs | rt | rd | shamt | funct |
| --- | --- | --- | --- | --- | --- | --- |
| Binary | 000000 | 01001 | 01010 | 01000 | 00000 | 100000 |
| Decimal | 0 | 9 | 10 | 8 | 0 | 32 |

Question 2.1 (Analyze)  Compare the decimal values of rs, rt and rd with the register numbers in Table 3. Which operand of the assembly text does each field hold? In which order are the three registers written in the assembly text, and in which order do their fields appear in the machine word?

rs corresponds to register $t1, rt to register $t2, and rd to register $t0. They are written in the order, destination, source 1, source 2. In 32-bit machine code, the fields appear in the order: source 1, source 2 , destination.

Question 2.2 (Understand)  Look at the row of li $v0, 10. Compare its Source and Basic columns. Is li an instruction that the hardware executes itself? What does the assembler do with it?

The field rs holds the first source operand $t1, rt holds the second source operand $t2, and rd holds the destination operand $t0. In the assembly text, the registers are written in the order of destination, source 1, and source 2. In the machine word, their fields appear in the order of source 1, source 2, and destination.

Screenshot of the Text Segment showing the row of add $t0, $t1, $t2:

Paste screenshot here

Step 2.3 – Energy used by a household (12 min)

Listing 3 loads four meter readings (in kWh) taken at the start of month 1, the start of month 2, the start of month 3 and the end of month 3. Complete the program so that $t0, $t1 and $t2 hold the energy used in months 1, 2 and 3, and $t3 holds the total. Use only add and sub.

Listing 3

# Listing 3 - energy used by a household (fictitious readings, kWh) # Variable Register # reading0 $s0 start of month 1 # reading1 $s1 start of month 2 # reading2 $s2 start of month 3 # reading3 $s3 end of month 3 # usage1 $t0 energy used in month 1 # usage2 $t1 energy used in month 2 # usage3 $t2 energy used in month 3 # total $t3 usage1 + usage2 + usage3 .text .globl main main: addi $s0, $zero, 1200 addi $s1, $zero, 1350 addi $s2, $zero, 1475 addi $s3, $zero, 1640 # >>> usage1 = reading1 - reading0 # >>> usage2 = reading2 - reading1 # >>> usage3 = reading3 - reading2 # >>> total = usage1 + usage2 + usage3 (two add instructions) li $v0, 10 syscall

Check your result With these readings, after the program has run, $t0, $t1, $t2 and $t3 must contain 150, 125, 165 and 440.

Screenshot of the Registers pane after the program has run:

Paste screenshot here

Question 2.3 (Analyze)  Calculate reading3 - reading0 by hand and compare it with total. Why must the two numbers be equal? Explain in terms of what the meter readings represent.

reading3 - reading0 = 1640 - 1200 = 440

Step 2.4 – Translate statements into instructions (13 min)

Table 9 lists four statements. In each of them the variables y, z and w live in $s0, $s1 and $s2 and start with the values 8, 5 and 3. Write the MIPS instructions for each statement in the yellow cells, using the destination register shown. Where an intermediate result is needed, use $t5 as a temporary register. PREDICT the result of each statement by hand before you test it.

Table 9. Statements to translate

| Statement | Destination | Your MIPS instructions | PREDICT (decimal) | OBSERVE (decimal) |
| --- | --- | --- | --- | --- |
| x1 = y + z + w | $t0 | add $t5, $s0, $s1 add $t0, $t5, $s2 | 16 | 16 |
| x2 = (y + z) - (w + 5) | $t1 | add $t5, $s0, $s1 addi $t1, $s2, 5 sub $t1, $t5, $t1 | 5 | 5 |
| x3 = y (copy) | $t2 | add $t2, $s0, $zero | 8 | 8 |
| x4 = z - y | $t3 | sub $t3, $s1, $s0 | -3 | -3 |

To test your answers, start a new file with the lines below, add your instructions at the marker, run the program and read $t0–$t3.

Listing 4

# Listing 4 - translating statements .text .globl main main: addi $s0, $zero, 8 # y = 8 addi $s1, $zero, 5 # z = 5 addi $s2, $zero, 3 # w = 3 # >>> instructions for x1, x2, x3 and x4 li $v0, 10 syscall

Question 2.4 (Analyze)  The value of x4 is negative. Show $t3 in hexadecimal and in decimal. What do the leading hexadecimal digits of a negative number look like, and what does this suggest about how the processor stores negative numbers?

$t3 hexadecimal is 0xfffffffd, decimal is -3, the leading F digits indicate that the processor stores negative numbers using two's complement representation.

EXPERIMENT 3. INPUT, OUTPUT AND SYSTEM CALLS  (50 minutes – 35 points)

Situation A technician types the readings into the smart meter, and the meter shows the energy used to the customer. A program running on a processor has no screen or keyboard of its own: it must ask the system to read and print on its behalf. This request is called a system call.

Background: program memory and text A MIPS program has two parts. The .text part holds the instructions. The .data part holds the data that the program uses, such as messages. In MARS the data part starts at address 0x10010000. A label is a name for an address. The directive .asciiz "text" stores the characters of a message followed by one extra byte of value 0, which marks the end of the message. The pseudo-instruction la $a0, label puts the address of the label into $a0.

Background: how to make a system call 1. Put the service number in $v0. 2. Put the argument, if any, in $a0. 3. Execute syscall. 4. If the service returns a result, read it from $v0. In MARS, the simulator plays the role of the operating system and performs the service. Table 10 lists the four services used in this LAB.

Table 10. System-call services used in this LAB

| Service | $v0 | Argument | Result |
| --- | --- | --- | --- |
| Print integer | 1 | $a0 = integer to print | – |
| Print string | 4 | $a0 = address of a string that ends with 0 | – |
| Read integer | 5 | – | the integer typed by the user, in $v0 |
| Exit program | 10 | – | – |

Step 3.1 – Print text and a number (12 min)

Type Listing 5, save it and assemble it. PREDICT exactly what will appear in the Run I/O pane.

Listing 5

# Listing 5 - print a message and an integer .data msg: .asciiz "Usage (kWh): " nl: .asciiz "\n" .text .globl main main: addi $t0, $zero, 150 li $v0, 4 # service 4: print string la $a0, msg # argument: address of msg syscall li $v0, 1 # service 1: print integer add $a0, $t0, $zero # argument: the number to print syscall li $v0, 4 # print a new line la $a0, nl syscall li $v0, 10 # service 10: exit syscall

PREDICT – text in the Run I/O pane:

Usage (kWh): 150

OBSERVE – run the program. Then open Settings → Show Labels Window (symbol table) and read the addresses of the two labels.

Table 11. Label addresses

| Label | Address (hex) |
| --- | --- |
| msg | 0x10010000 |
| nl | 0x1001000e |

Screenshot of the Run I/O pane after the program has run:

Paste screenshot here

Question 3.1 (Understand)  Which register told the system which service was wanted, and which register carried what to print? What would happen to the meaning of $a0 if you changed the service number?

The $v0 register informs the system which service to perform, while the $a0 register holds the content or arguments to be printed; if the service number is changed, the meaning of $a0 will change accordingly.

Question 3.2 (Apply)  Subtract the address of msg from the address of nl. Count the characters of the message Usage (kWh):  (including spaces) and compare with your result. How do you explain the difference between the two numbers?

0x1001000e - 0x10010000 = 14 (bytes). The string "Usage (kWh): " contains 13 characters including spaces and is declared using the .asciiz directive. The system automatically appends a null byte \0 at the end to mark the termination of the string. The total space used is 13 + 1 = 14 bytes.

Step 3.2 – Read two numbers and compute the usage (15 min)

Listing 6 reads the previous meter reading from the keyboard and stores it in $s0. Complete the program so that it also reads the current reading into $s1, computes usage = current - previous in $t0, and prints the result in the form shown below the listing.

Listing 6

# Listing 6 - energy used between two readings .data prompt1: .asciiz "Enter previous reading (kWh): " prompt2: .asciiz "Enter current reading (kWh): " label: .asciiz "Usage (kWh): " newline: .asciiz "\n" .text .globl main main: # --- read the previous reading into $s0 --- li $v0, 4 la $a0, prompt1 syscall li $v0, 5 # service 5: read integer, result in $v0 syscall add $s0, $v0, $zero # keep the value in $s0 # >>> read the current reading into $s1 (print prompt2, then service 5) # >>> usage = current - previous, into $t0 # >>> print label, then the usage (service 1), then newline li $v0, 10 syscall

Check your result Run the program and type 1200 and 1350 when asked. The last line in the Run I/O pane must be Usage (kWh): 150. Type each number in the Run I/O pane and press Enter.

Screenshot of the Run I/O pane for this test:

Paste screenshot here

Step 3.3 – Three-month summary with your own data (15 min)

Extend your program so that it reads four readings in order (reading0 to reading3) and then prints the usage of each month and the total. Start from the .data section below and keep the register plan of Listing 3 ($s0–$s3 for the readings, $t0–$t3 for the results). The output must have this form:

Month 1: <usage1> Month 2: <usage2> Month 3: <usage3> Total: <total>

Listing 7 (data section)

.data prompt: .asciiz "Enter reading (kWh): " m1: .asciiz "Month 1: " m2: .asciiz "Month 2: " m3: .asciiz "Month 3: " tot: .asciiz "Total: " nl: .asciiz "\n"

First test the program with the readings 1200, 1350, 1475 and 1640; the four results must be 150, 125, 165 and 440. Then take D, the last digit of your student ID, and look up your own readings in Table 12. PREDICT the four results by hand, run the program with your readings, and record what you observe.

Table 12. Personal readings (D = last digit of the student ID)

| D | reading0 | reading1 | reading2 | reading3 |
| --- | --- | --- | --- | --- |
| 0 | 1000 | 1120 | 1220 | 1360 |
| 1 | 1020 | 1143 | 1248 | 1390 |
| 2 | 1040 | 1166 | 1276 | 1420 |
| 3 | 1060 | 1189 | 1304 | 1450 |
| 4 | 1080 | 1212 | 1332 | 1480 |
| 5 | 1100 | 1235 | 1360 | 1510 |
| 6 | 1120 | 1258 | 1388 | 1540 |
| 7 | 1140 | 1281 | 1416 | 1570 |
| 8 | 1160 | 1304 | 1444 | 1600 |
| 9 | 1180 | 1327 | 1472 | 1630 |

Table 13. Results for your own readings

| My D | Result | Month 1 | Month 2 | Month 3 | Total |
| --- | --- | --- | --- | --- | --- |
| 5 | PREDICT | 135 | 125 | 150 | 410 |
| 5 | OBSERVE | 135 | 125 | 150 | 410 |

Screenshot of the Run I/O pane with your own readings:

Paste screenshot here

Step 3.4 – Break it on purpose (8 min)

Programmers learn a great deal from errors. For each experiment in Table 14, PREDICT what will happen, make the change, run the program and OBSERVE the result. Restore the program afterwards.

Table 14. Controlled experiments

| Experiment | PREDICT | OBSERVE |
| --- | --- | --- |
| (a) Run your Step 3.2 program and enter the readings in the wrong order: 1350 first, then 1200. | 1200 - 1350 = -150 | Usage (kWh): -150 |
| (b) In Listing 5, delete the last two instructions (li $v0, 10 and syscall) and run it. | The program does not terminate cleanly, it continues to execute subsequent memory locations within the text segment until an error occurs. | MARS reports an execution error when a program runs beyond the valid range of instructions within the text segment of memory. |
| (c) In Listing 5, change the service number before printing the integer from 1 to 4, and run it. | MARS sẽ hiểu lệnh là in chuỗi và coi giá trị trong $a0 là một địa chỉ bộ nhớ, dẫn tới lỗi bộ nhớ. | Runtime exception at 0x00400018: address out of range 0x00000096 |

Question 3.3 (Analyze)  Explain your observation in experiment (c). What did the system try to treat as an address, and why did the program fail?

Service 4 (print string) expects register $a0 to contain a memory address pointing to a null-terminated string. Instead, $a0 held the integer value 150 (hexadecimal 0x00000096). The system tried to treat 0x00000096 as a memory address to fetch text from. The program failed because 0x00000096 is far outside the valid data segment (which starts at 0x10010000), causing an "address out of range" runtime exception.

PART 4. WRAP-UP AND SUBMISSION  (15 minutes – 15 points)

Question 4.1 (Understand)  In your own words, describe how the statement a = b + c becomes something the processor can execute. Use the terms register, assembly instruction, assembler and machine code.

The statement a=b+c maps variables to registers to form an assembly instruction (add $t0, $t1, $t2). The assembler then translates this instruction into 32-bit machine code for the processor to execute.

Question 4.2 (Apply)  Write the instructions that print the integer held in $t5 followed by a new line, assuming that a string labelled nl exists in the .data section.

# in so nguyen trong thanh ghi $t5 li $v0, 1 add $a0, $t5, $zero syscall # in ky tu xuong dong li $v0, 4 la $a0, nl syscall

Question 4.3 (Evaluate)  Your Step 3.3 program contains several almost identical blocks of instructions. In two or three sentences, suggest a feature that would make such a program shorter, and say what the processor would need to be able to do to support it.

Loops or subroutines can be used to reuse repeated instruction blocks and make the program shorter. To support this, the processor needs jump instructions (such as bne, jal, jr) and a return address register ($ra).

Table 15. Submission checklist

| ☐ | Item |
| --- | --- |
| ☐ | Files Lab01_<StudentID>_TN2.asm and Lab01_<StudentID>_TN3.asm are saved with the correct names. |
| ☐ | Every file begins with a comment block (name, student ID, purpose) and every program assembles without errors. |
| ☐ | Every # >>> marker has been replaced by your own code and the register plan is documented in comments. |
| ☐ | This document is complete: all yellow cells are filled and every dashed frame contains a screenshot. |
| ☐ | The .asm files are pushed to your GitHub repository, and the repository link is written in the information table on the first page. |
| ☐ | The document and the repository link are submitted in the way announced by your instructor. |

APPENDIX. INSTRUCTION REFERENCE

Table 16. Instructions used in this LAB

| Instruction | Example | Meaning |
| --- | --- | --- |
| add | add $t0, $t1, $t2 | $t0 = $t1 + $t2 |
| sub | sub $t0, $t1, $t2 | $t0 = $t1 - $t2 |
| addi | addi $t0, $t1, 20 | $t0 = $t1 + 20 |
| syscall | syscall | ask the system to perform the service whose number is in $v0 |
| li (pseudo) | li $t0, 7 | $t0 = 7; the assembler turns it into a real instruction |
| la (pseudo) | la $a0, label | $a0 = address of label; the assembler turns it into real instructions |